# EPSG, CRS and Coordinate Transformation Guide for Java Developers

## 1. Introduction

This document explains:

- What is a Coordinate Reference System (CRS)

- What is an EPSG code

- How to identify source and target coordinate systems

- How to determine the correct EPSG code

- Common coordinate systems used in GIS applications

- Java examples using Proj4J and GeoTools

- Real-world example using North Carolina State Plane coordinates

# 2. What is a CRS?

CRS stands for Coordinate Reference System.

A CRS defines how coordinates relate to locations on the Earth’s surface.

A CRS consists of:

- Datum

- Projection

- Units

- Coordinate Axes

Example:

EPSG:2264

Datum: NAD83

Projection: Lambert Conformal Conic

Units: US Survey Feet

Area: North Carolina

Without CRS information, coordinate values are meaningless.

Example:

X = 2121114.512 Y = 947404.711

These numbers alone cannot determine a location because many coordinate systems can produce similar values.

# 3. What is an EPSG Code?

EPSG stands for:

European Petroleum Survey Group

An EPSG code uniquely identifies a coordinate reference system.

Examples:

EPSG:4326 -> WGS84 Geographic Coordinates

EPSG:4269 -> NAD83 Geographic Coordinates

EPSG:3857 -> Web Mercator

EPSG:2264 -> NAD83 North Carolina State Plane (US Feet)

EPSG:32119 -> NAD83 North Carolina State Plane (Meters)

EPSG:32617 -> UTM Zone 17N (WGS84)

EPSG:26917 -> UTM Zone 17N (NAD83)

Think of EPSG codes as IDs for coordinate systems.

# 4. Geographic vs Projected Coordinate Systems

## Geographic Coordinate Systems

Coordinates are represented as:

Latitude Longitude

Example:

Latitude: 36.352301627

Longitude: -78.588754655

Examples:

EPSG:4326 EPSG:4269

## Projected Coordinate Systems

Coordinates are represented as:

X (Easting) Y (Northing)

Example:

Easting: 2121114.512

Northing: 947404.711

Examples:

EPSG:2264 EPSG:32119 EPSG:3857

# 5. Common Datums

## WGS84

EPSG:4326

Used by:

- GPS

- Google Maps

- OpenStreetMap

- GeoJSON

## NAD83

EPSG:4269

Used primarily in:

- United States

- Government GIS datasets

- NOAA

- Utility companies

## NAD27

EPSG:4267

Older US coordinate system.

Still found in legacy datasets.

# 6. Common GIS Coordinate Systems

| EPSG | Description |
| --- | --- |
| 4326 | WGS84 Latitude/Longitude |
| 4269 | NAD83 Latitude/Longitude |
| 3857 | Web Mercator |
| 2264 | North Carolina State Plane (Feet) |
| 32119 | North Carolina State Plane (Meters) |
| 32617 | UTM Zone 17N WGS84 |
| 26917 | UTM Zone 17N NAD83 |

# 7. How to Identify Source CRS

This is the most important step.

## Step 1

Determine coordinate type.

If values look like:

36.352 -78.588

Then source is likely Geographic.

If values look like:

2121114 947404

Then source is likely Projected.

## Step 2

Look for clues:

- State Plane

- UTM

- Zone

- Datum

- Units

## Step 3

Search EPSG databases.

Examples:

“North Carolina State Plane NAD83 Feet”

“UTM Zone 17N”

“NAD83 Geographic”

# 8. Source CRS Decision Tree

Do you already have Latitude/Longitude?

YES

WGS84 -> EPSG:4326

NAD83 -> EPSG:4269

NO

Do you have UTM coordinates?

YES

Find UTM Zone

Example:

Zone 17N -> EPSG:32617

Zone 18N -> EPSG:32618

NO

Do you have State Plane coordinates?

YES

Identify:

- State

- Datum

- Units

Then search EPSG.

Example:

North Carolina NAD83 US Survey Feet

Result:

EPSG:2264

# 9. How to Determine Target CRS

Target CRS depends on desired output.

| Requirement | Target CRS |
| --- | --- |
| Latitude/Longitude | EPSG:4326 |
| NAD83 Latitude/Longitude | EPSG:4269 |
| Google Maps | EPSG:4326 |
| GeoJSON | EPSG:4326 |
| UTM Zone 17N | EPSG:32617 |
| State Plane NC | EPSG:2264 |

# 10. Real World Example

Input Information:

Coordinate Type: SPC

Zone: NC-3200

Datum: NAD83(2011)

Units: US Survey Feet

Coordinates:

Northing: 947404.711

Easting: 2121114.512

Determine Source CRS

Search:

North Carolina State Plane NAD83 Feet

Result:

EPSG:2264

Determine Target CRS

Desired Output:

Latitude Longitude

Datum:

NAD83

Result:

EPSG:4269

Transformation

EPSG:2264 -> EPSG:4269

Output:

Latitude: 36.352301627

Longitude: -78.588754655

# 11. Java Example Using Proj4J

Maven Dependency

org.locationtech.proj4j

org.locationtech.proj4j-epsg

Version:

1.3.0

Java Code

CRSFactory factory = new CRSFactory();

CoordinateReferenceSystem sourceCRS = factory.createFromName(“EPSG:2264”);

CoordinateReferenceSystem targetCRS = factory.createFromName(“EPSG:4269”);

CoordinateTransformFactory ctFactory = new CoordinateTransformFactory();

CoordinateTransform transform = ctFactory.createTransform(sourceCRS, targetCRS);

ProjCoordinate source = new ProjCoordinate(2121114.512, 947404.711);

ProjCoordinate target = new ProjCoordinate();

transform.transform(source, target);

System.out.println(“Latitude :” + target.y); System.out.println(“Longitude:” + target.x);

# 12. Java Example Using GeoTools

CoordinateReferenceSystem sourceCRS = CRS.decode(“EPSG:2264”);

CoordinateReferenceSystem targetCRS = CRS.decode(“EPSG:4269”);

MathTransform transform = CRS.findMathTransform(sourceCRS, targetCRS, true);

DirectPosition2D source = new DirectPosition2D(2121114.512, 947404.711);

DirectPosition2D target = new DirectPosition2D();

transform.transform(source, target);

System.out.println(target.y); System.out.println(target.x);

# 13. GeoTools vs Proj4J

| Feature | Proj4J | GeoTools |
| --- | --- | --- |
| Easy Setup | Yes | No |
| Lightweight | Yes | No |
| Full EPSG Support | Limited | Yes |
| Datum Transformations | Limited | Excellent |
| Production GIS Work | Moderate | Recommended |

# 14. Troubleshooting Guide

## Wrong Location

Cause:

Incorrect source EPSG

## Latitude and Longitude Swapped

Cause:

Axis order issue

Solution:

Print target.y as latitude

Print target.x as longitude

## Location Shifted By Few Meters

Cause:

Wrong datum

Example:

WGS84 vs NAD83

## Coordinates Thousands Of Miles Away

Cause:

Feet vs Meters mismatch

## Conversion Produces NaN

Cause:

Invalid CRS definition

Verify EPSG code.

# 15. Useful Websites

EPSG Registry https://epsg.org

EPSG Lookup https://epsg.io

NOAA Coordinate Conversion Tool https://geodesy.noaa.gov/NCAT

Spatial Reference Database https://spatialreference.org

# 16. Best Practices

- Always identify source CRS first.

- Never assume coordinates are WGS84.

- Verify units (feet vs meters).

- Verify datum.

- Validate results using NOAA or EPSG.io.

- Prefer GeoTools for enterprise GIS applications.

- Document CRS assumptions in code and APIs.

# 17. Key Takeaways

EPSG identifies coordinate systems.

CRS defines how coordinates relate to Earth.

Source CRS determines where coordinates come from.

Target CRS determines the desired output format.

For the North Carolina example:

Source: EPSG:2264

Target: EPSG:4269

Transformation:

Northing/Easting -> Latitude/Longitude

Result:

36.352301627 -78.588754655

This matches NOAA NCAT conversion results.